ESP32 Project Setup Guide

Plain ESP-IDF + Wokwi (wokwi.toml + diagram.json), driven from Git Bash or VS Code. No PlatformIO.

# 1. Overview

This guide records the working setup, from a fresh machine to creating a new project and simulating it in Wokwi. You can drive the same install two ways, and they can be mixed freely because both read and write the same build folder:

- Git Bash: an idf helper function replaces idf.py.

- VS Code: the Espressif IDF extension (command palette and status bar buttons) plus the Wokwi extension.

| Item | Value |
| --- | --- |
| ESP-IDF | C:\esp\esp-idf (v5.2.3) |
| Tools | C:\dev\esp-tools |
| Python environment | C:\dev\esp-tools\python_env\idf5.2_py3.14_env |
| Projects folder | C:\Uni\EmbeddedSystemsProjects\ |
| Simulator | Wokwi for VS Code. It reads the build output, so no board or COM port is needed. |
| Rule | No spaces in any path. ESP-IDF builds fail in strange ways otherwise. |

# 2. One-time machine setup

## 2.1 Prerequisites

- Git for Windows (provides Git Bash).

- Python on the Windows PATH. 3.11 or 3.12 is the safest choice. 3.14 worked in this setup but is newer than ESP-IDF v5.2 was tested with.

- VS Code with two extensions: Espressif IDF and Wokwi Simulator.

## 2.2 Install ESP-IDF (Git Bash)

mkdir -p /c/esp && cd /c/esp

git clone -b v5.2.3 --recursive https://github.com/espressif/esp-idf.git

MSYS_NO_PATHCONV=1 cmd /c "set MSYSTEM=&& set IDF_TOOLS_PATH=C:\dev\esp-tools&& cd /d C:\esp\esp-idf && install.bat esp32"

Why set MSYSTEM=: the ESP-IDF .bat scripts refuse to run when the MSYSTEM variable is defined, and Git Bash defines it. Clearing it inside cmd gets past the check. There must be no space before each && that follows a set, or the space becomes part of the value.

For a different chip (for example esp32s3), rerun the last command with that chip name in place of esp32.

## 2.3 Add the idf helper (Git Bash)

cat >> ~/.bashrc << 'EOF'

idf() {

MSYS_NO_PATHCONV=1 cmd /c "set MSYSTEM=&& set IDF_TOOLS_PATH=C:\dev\esp-tools&& call C:\esp\esp-idf\export.bat >nul && idf.py $*"

}

EOF

source ~/.bashrc

Use idf anywhere you would normally type idf.py. Each call re-activates the environment, which adds a second or two.

## 2.4 Verify

idf --version        # expect: ESP-IDF v5.2.3

Lines saying "Not using an unsupported version of tool cmake/ninja/ccache" are harmless. ESP-IDF is ignoring the newer copies on your PATH and using its own.

## 2.5 Connect the VS Code ESP-IDF extension

- Press Ctrl+Shift+P and run ESP-IDF: Configure ESP-IDF extension.

- Choose Use existing setup.

- Set the ESP-IDF path to C:\esp\esp-idf and the tools path to C:\dev\esp-tools, then finish the wizard.

- Open your project folder. IntelliSense errors such as freertos/FreeRTOS.h not found clear once the project has been built (the build produces compile_commands.json). If they linger, run ESP-IDF: Add vscode configuration folder, then Developer: Reload Window.

VSCODE Intellisense Failure.

Pres Ctrl + Shift +P :Then enter ESP-IDF: Add vscode configuration folder

Inside VS Code's ESP-IDF terminal (ESP-IDF: Open ESP-IDF Terminal) the environment is already active, so type idf.py there, not idf.

## 2.6 Activate Wokwi (first run only)

Press Ctrl+Shift+P, run Wokwi: Request a New License, and complete the free license activation in your browser.

# 3. Creating a new project

## Option A: Git Bash

cd /c/Uni/EmbeddedSystemsProjects

idf create-project my_project

cd my_project

code .                  # open in VS Code

idf set-target esp32    # once per project; creates sdkconfig

idf build               # first build is slow, later ones are incremental

To start from an example instead of an empty project, for example blink:

cp -r /c/esp/esp-idf/examples/get-started/blink ./my_project

## Option B: VS Code

- Ctrl+Shift+P then ESP-IDF: New Project, choose a name and a location under C:\Uni\EmbeddedSystemsProjects\. Or use ESP-IDF: Show Examples Projects and create from the blink example.

- Run ESP-IDF: Set Espressif Device Target and choose esp32.

- Run ESP-IDF: Build Your Project, or click the build icon in the status bar.

## Sample LED blink (main/main.c)

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#include "driver/gpio.h"

#define LED_GPIO GPIO_NUM_2

void app_main(void)

{

gpio_reset_pin(LED_GPIO);

gpio_set_direction(LED_GPIO, GPIO_MODE_OUTPUT);

while (1) {

gpio_set_level(LED_GPIO, 1);

vTaskDelay(pdMS_TO_TICKS(500));

gpio_set_level(LED_GPIO, 0);

vTaskDelay(pdMS_TO_TICKS(500));

}

}

# 4. Wokwi files (project root)

Put both files next to the top-level CMakeLists.txt.

## wokwi.toml

[wokwi]

version = 1

firmware = 'build/flasher_args.json'

elf = 'build/my_project.elf'

Replace my_project with the name in project(...) inside the top-level CMakeLists.txt. The ELF is named after that project name.

## diagram.json (ESP32 + LED on GPIO 2)

{

"version": 1,

"author": "",

"editor": "wokwi",

"parts": [

{ "type": "board-esp32-devkit-c-v4", "id": "esp", "top": 0, "left": 0, "attrs": {} },

{ "type": "wokwi-resistor", "id": "r1", "top": -20, "left": 130, "rotate": 90, "attrs": { "value": "220" } },

{ "type": "wokwi-led", "id": "led1", "top": -70, "left": 190, "attrs": { "color": "red" } }

],

"connections": [

["esp:TX", "$serialMonitor:RX", "", []],

["esp:RX", "$serialMonitor:TX", "", []],

["esp:2", "r1:1", "green", []],

["r1:2", "led1:A", "green", []],

["led1:C", "esp:GND.1", "black", []]

]

}

The GPIO in your code must match the pin in the diagram: esp:2 means GPIO 2. Part positions are only layout, so drag them around in the Wokwi editor if they overlap. You can also build the circuit on wokwi.com and paste its JSON into this file.

# 5. Command reference: Git Bash vs VS Code

| Task | Git Bash | VS Code (Ctrl+Shift+P) |
| --- | --- | --- |
| Check version | idf --version | In ESP-IDF terminal: idf.py --version |
| Set chip | idf set-target esp32 | ESP-IDF: Set Espressif Device Target |
| Build | idf build | ESP-IDF: Build Your Project |
| Settings menu | idf menuconfig | ESP-IDF: SDK Configuration Editor |
| Clean rebuild | idf fullclean | ESP-IDF: Full Clean Project |
| Start simulator | (VS Code only) | Wokwi: Start Simulator |
| Stop simulator | (VS Code only) | Wokwi: Stop Simulator |
| Pick COM port | (pass -p COM3) | ESP-IDF: Select Port to Use |
| Flash real board | idf -p COM3 flash | ESP-IDF: Flash Your Project |
| Serial monitor (real board) | Unreliable in Git Bash | ESP-IDF: Monitor Your Device |
| Open IDF terminal | (not needed) | ESP-IDF: Open ESP-IDF Terminal |

Command names can differ slightly between extension versions. Typing "ESP-IDF" or "Wokwi" in the command palette lists everything available.

# 6. Daily workflow

- Edit your code in main/.

- Build: idf build in Git Bash, or ESP-IDF: Build Your Project in VS Code.

- Run Wokwi: Stop Simulator, then Wokwi: Start Simulator, so it loads the new firmware. Wokwi does not pick up changes on its own.

- Watch the LED and the serial output in the Wokwi panel.

The first build compiles all of ESP-IDF and takes a few minutes. After that, builds are incremental and take seconds. A full rebuild happens only after set-target, fullclean, deleting build, or changing sdkconfig settings that affect many components.

Wokwi never needs flashing or a COM port. Flash only when you move to real hardware: idf -p COM3 flash. If the upload stalls at "Connecting......", hold the BOOT button on the board.

# 7. New project checklist

- Create the project under C:\Uni\EmbeddedSystemsProjects\ (no spaces in the path).

- idf set-target esp32 (or the VS Code equivalent).

- Write the code in main/main.c.

- idf build and confirm it ends with "Project build complete".

- Add wokwi.toml, with the ELF name matching the project name.

- Add diagram.json, with the LED pin matching the GPIO in the code.

- Wokwi: Start Simulator.

# 8. Troubleshooting

| Symptom | Cause and fix |
| --- | --- |
| "This .bat file is for Windows CMD.EXE shell only" | MSYSTEM is set by Git Bash. Use the set MSYSTEM=&& prefix shown in section 2. |
| cmd prints a Windows banner and does nothing | Wrong flag. Use cmd /c (single slash) with MSYS_NO_PATHCONV=1, not //c. |
| Wokwi cannot find the firmware | The build has not finished, or the paths in wokwi.toml are wrong. Check the ELF name. |
| Wokwi shows old behaviour | Stop and restart the simulator after every build. |
| LED does not light in the simulator | The GPIO in the code differs from the pin wired in diagram.json. |
| freertos/FreeRTOS.h not found in VS Code | Configure the extension with Use existing setup, build once, then reload the window. |
| Odd Python errors during build | Python 3.14 is newer than ESP-IDF v5.2 supports. Install Python 3.11 or 3.12 and rerun the install. |
| Build output only mentions esp32 tools, but you use another chip | Rerun the install command with your chip name (for example esp32s3). |